# FALCON SYSTEM INTEGRATION SPECIFICATION

Hardware Platform: NVIDIA Jetson Orin (Ubuntu ARM64) Sensor Payload: Intel RealSense D435i (RGB-D + IMU) & TI IWR6843ISK (60GHz mmWave Radar) Application: Short-to-Mid Range People Counting & Surveillance

## 1. Intel RealSense D435i Subsystem

### SDK Compatibility & Build Configuration

For the Jetson Orin (running JetPack 5.x or 6.x), the recommended SDK is librealsense2 v2.55.1. Newer beta versions currently present regressions with the JetPack 6.x hid-sensor kernel drivers for the IMU.

- Build Flags: To compile directly on the Orin, build from source using CMake with the following critical flags:

  - -DBUILD_WITH_CUDA=true (Offloads depth alignment and point-cloud generation to the Orin’s Ampere GPU, drastically reducing CPU load).

  - -DFORCE_RSUSB_BACKEND=true (Bypasses the need to patch the L4T Linux kernel for UVC payload handling, which is highly recommended for JetPack 6.0+ stability).

### Stream Configuration & Optimal Settings

For optimal human detection, align the depth and color frame spaces to minimize warping overhead:

- Depth Stream: 848 x 480 at 30 FPS, Format: Z16. (This is the hardware sweet spot for the D435i ASIC stereoscopic matching).

- Color Stream: 848 x 480 at 30 FPS, Format: BGR8.

- Infrared (IR): 848 x 480 at 30 FPS, Format: Y8 (Optional: highly useful for verifying structured light patterns or low-light surveillance).

- IMU: Accelerometer at 200 Hz, Gyroscope at 400 Hz (MOTION_XYZ32F).

### Depth Post-Processing Pipeline

To optimize depth maps for human detection and suppress typical stereo artifacts:

- Decimation Filter: Downsamples the depth map (magnitude 2 or 3) to reduce computational load and smooth noise.

- Spatial Filter: Edge-preserving smoothing (Alpha: 0.5, Delta: 20, Iterations: 2).

- Temporal Filter: Averages consecutive frames (Alpha: 0.4, Delta: 20). Critical for stationary person tracking.

- Hole-Filling Filter: Nearest-neighbor heuristic to patch gaps in depth maps typical of human clothing (which absorbs IR).

### Calibration Programmatic Access

Intrinsics and extrinsics are read directly via the SDK. You will need the extrinsics to map Depth pixels to Color pixels (get_extrinsics_to()), and the depth intrinsics (rs2_intrinsics) to deproject 2D pixels to 3D camera-space coordinates.

## 2. TI IWR6843ISK mmWave Radar Subsystem

### SDK Version & Data Capture Method

Use TI mmWave SDK 3.5.x coupled with the Radar Toolbox (formerly Industrial Toolbox).

- Data Capture: You must use UART. The DCA1000EVM is exclusively for capturing raw ADC data (IF signals) over Ethernet for offline processing. For embedded inference on the Jetson, the IWR6843’s onboard C674x DSP and ARM R4F must process the raw RF into a 3D point cloud and output it via UART.

### Hardware & Antenna Configuration

The IWR6843ISK utilizes a 3Tx / 4Rx (MIMO) antenna configuration. When Tx multiplexing (TDM-MIMO) is configured correctly in the chirp profile, this yields a virtual antenna array of 12 elements, providing both Azimuth and Elevation angle estimation (essential for 3D human tracking).

### Chirp Configuration (0.5m – 8m Optimization)

To optimize for 8-meter human detection, the .cfg file must tune the frequency slope and IF sampling rate.

- profileCfg: Configured for a sweep bandwidth that yields ~4-5 cm range resolution.

- frameCfg: Set to 50ms frame periodicity (20 FPS) to balance DSP processing time and temporal latency.

### UART Interface & TLV Format

The IWR6843 interfaces via two USB serial endpoints (enumerated via the carrier board’s CP2105 silicon):

- CLI Port (115200 baud): Used at startup to send the .cfg text file line-by-line.

- Data Port (921600 baud): Streams the binary output.

- TLV Structure: The stream begins with an 8-byte Magic Word (02 01 04 03 06 05 08 07), followed by a 40-byte Frame Header containing total packet length. Data is packed in Type-Length-Value (TLV) blocks. Key types include:

  - Type 6: Point Cloud (X, Y, Z, Doppler Velocity, SNR).

  - Type 7: Target List (Tracked IDs, Bounding Boxes, Kinematics).

- Parsers: Leverage the Python ecosystem. The kirkster96/IWR6843-Read-Data-Python-MMWAVE-SDK or SuperTigerPro/IWR6843_TLVS repositories contain battle-tested byte-unpacking (struct.unpack) logic for these magic words and IEEE 754 float conversions.

## 3. Sensor Fusion & Integration Architecture (No ROS)

### Coordinate Frame Alignment

The sensors have fundamentally different coordinate spaces:

- IWR6843: Cartesian (X: Right, Y: Forward/Depth, Z: Up)

- RealSense: Right-Handed Vision (X: Right, Y: Down, Z: Forward/Depth)

- Base Rotation: To map Radar coordinates (Pr) to Camera coordinates (Pc), apply the rotation matrix Rbase: Xc=Xr,Yc=−Zr,Zc=Yr.

### Spatial Calibration

To compute the exact Rigid Transform (R,T) including physical offsets:

- Mount a metallic Corner Reflector (high radar cross-section) within the FOV.

- Record the 3D position from the Radar (it will be the point with the absolute highest SNR).

- Use the RealSense Depth stream to isolate the physical apex of the reflector and deproject it to 3D space.

- Collect N points at varying distances and use Singular Value Decomposition (SVD) or a Least-Squares estimator to solve for the rigid transformation matrix (Pc=R⋅Pr+T).

### Temporal Synchronization & Threading Pipeline

Since the devices operate at different frequencies (Camera ~30FPS, Radar ~15-20FPS) with no hardware trigger sync available between them:

- Use Python multiprocessing or threading.Thread with thread-safe queue.Queue.

- Thread 1: Radar UART polling. Upon detecting the magic word, parse the TLV, attach a UNIX epoch timestamp, and push to the Radar Queue.

- Thread 2: RealSense polling (pipeline.wait_for_frames()). Attach a timestamp and push to the Camera Queue.

- Thread 3 (Fusion): Event-triggered by the slower Radar thread. When a Radar frame arrives, it pops the closest temporally matching Camera frame using a Nearest-Neighbor timestamp approach (maximum acceptable delta: ~16ms).

## 4. People Counting Configuration

### IWR6843 Tracking Parameters

- Detection Zones: Configured via boundaryBox and presenceBoundaryBox in the .cfg. For an 8m room, set boundaryBox -4 4 0 8.1 -0.5 3 (X: ±4m, Y: 0 to 8m, Z: -0.5m to 3m).

- CFAR Thresholds: Use CASO-CFAR (Cell Averaging Smallest Of). dynamicRACfarCfg should be aggressively tuned to filter static multipath clutter while allowing micro-Doppler signatures (limbs moving).

- Clustering: TI’s Group Tracker uses a modified DBSCAN algorithm. Ensure trackingCfg specifies human-sized allocation parameters (e.g., max radial velocity, typical point cloud density for a torso). Refer to TI’s 3D People Tracking lab (lab0012 equivalents) in the Radar Toolbox.

### RealSense Depth ROI Filtering

Project the radar’s high-confidence tracked target (X, Y, Z) into the camera’s 2D pixel space u,v using the camera intrinsics. Create a bounding box (e.g., 100x200 pixels) around u,v. Verify the human presence by comparing the median depth value within this Camera ROI against the highly accurate absolute range reported by the Radar.

## 5. System Output & Deliverables

### Hardware Connection Summary

- RealSense D435i: USB 3.2 Type-C to Jetson USB 3.x Host (Must be enumerated as USB 3.2 for 30FPS uncompressed depth; verify with lsusb -t). Power draw: ~3.5W.

- IWR6843ISK: Mounted on MMWAVEICBOOST or standalone carrier. Connect the “XDS110” micro-USB port to Jetson. This enumerates /dev/ttyUSB0 (CFG) and /dev/ttyUSB1 (DATA). Power: 5V / 3A DC barrel jack.

### System Architecture Diagram

[RealSense D435i]                       [TI IWR6843ISK Radar]
       | USB 3.2 Type-C                           | Micro-USB (Carrier Board)
       v                                          v
+------------------------------------------------------------------+
| NVIDIA Jetson Orin (Ubuntu ARM64)                                |
|                                                                  |
| +---------------------+ +----------------------------+           |
| | librealsense2 SDK | | PySerial UART + TLV Parser   |           |
| | Polling Thrd (30Hz) | | Polling Thrd (15-20Hz)     |           |
| +---------+-----------+ +-------------+--------------+           |
|           | {RGB, Depth, IMU}         | {Point Cloud}            |
|           v                           v                          |
| +------------------------------------------------------------+   |
| |                      Fusion Engine                         |   |
| | 1. Timestamp Sync (Radar Event pops closest Camera frame)  |   |
| | 2. Matrix Transform (P_cam = R * P_radar + T)              |   |
| | 3. ROI Masking (Project Radar track to Depth Map)          |   |
| +-----------------------------+------------------------------+   |
|                               |                                  |
|                               v                                  |
|               [Output: Fused People Coordinates]                 |
+------------------------------------------------------------------+

### Master Configuration File Template (IWR6843 .cfg)

Save as falcon_people_counting.cfg (Tune Scenery parameters as needed):

sensorStop
flushCfg
dfeDataOutputMode 1
channelCfg 15 7 0
adcCfg 2 1
adcbufCfg -1 0 1 1 1
profileCfg 0 60.6 30 10 62 0 0 24 1 128 2500 0 0 30
chirpCfg 0 0 0 0 0 0 0 1
chirpCfg 1 1 0 0 0 0 0 4
chirpCfg 2 2 0 0 0 0 0 2
frameCfg 0 2 32 0 50 1 0
dynamicRACfarCfg -1 4 4 2 2 8 12 4 8 5.00 8.00 0.40 1 1
staticRACfarCfg -1 6 2 2 2 8 8 6 4 8.00 15.00 0.30 0 0
dynamicRangeAngleCfg -1 0.75 0.0010 1 0
dynamic2DAngleCfg -1 3 0.0300 1 0 1 0.30 0.85 8.00
staticRangeAngleCfg -1 0 1 1
static2DAngleCfg -1 1 0.1200 1 0 1 0.30 0.85 8.00
fineResolutionCfg 1 0
trackingCfg 1 2 250 20 52 82 50 90
boundaryBox -4 4 0 8 -0.5 3
presenceBoundaryBox -4 4 0.5 8 -0.5 3
sensorPosition 2 0 15 0 0
sensorStart

### RealSense Pipeline Initialization (Python)

import pyrealsense2 as rs

def init_realsense():
    pipeline = rs.pipeline()
    config = rs.config()

    # Hardware streams optimized for Jetson & alignment
    config.enable_stream(rs.stream.depth, 848, 480, rs.format.z16, 30)
    config.enable_stream(rs.stream.color, 848, 480, rs.format.bgr8, 30)
    config.enable_stream(rs.stream.accel, rs.format.motion_xyz32f, 200)
    config.enable_stream(rs.stream.gyro, rs.format.motion_xyz32f, 400)

    # Start and fetch device
    profile = pipeline.start(config)
    depth_sensor = profile.get_device().first_depth_sensor()
    
    # 3 = High Density Preset (Good for human targets/clothing)
    depth_sensor.set_option(rs.option.visual_preset, 3) 

    # Initialize processing blocks
    filters = {
        'decimation': rs.decimation_filter(),
        'spatial': rs.spatial_filter(),
        'temporal': rs.temporal_filter(),
        'hole_filling': rs.hole_filling_filter(),
        'align': rs.align(rs.stream.color) # Align depth to color viewport
    }
    
    # Fetch intrinsic/extrinsic data for sensor fusion math
    depth_profile = rs.video_stream_profile(profile.get_stream(rs.stream.depth))
    color_profile = rs.video_stream_profile(profile.get_stream(rs.stream.color))
    depth_intrinsics = depth_profile.get_intrinsics()
    depth_to_color_extrin = depth_profile.get_extrinsics_to(color_profile)
    
    return pipeline, filters, depth_intrinsics

pipeline, filters, intrinsics = init_realsense()

### Key Reference Repositories & Docs

- librealsense Source & Build Guides: github.com/IntelRealSense/librealsense (Refer specifically to installation_jetson.md)

- TI mmWave Radar Toolbox: dev.ti.com/tirex/explore/node?node=A__AMOPFE7Oh3d8zwpgSQZ9Zg__radar_toolbox__1AslXXD__LATEST (View 3D People Tracking release notes for tracker implementation details).

- Python TLV Parser Architecture: github.com/kirkster96/IWR6843-Read-Data-Python-MMWAVE-SDK (Critical reference for byte-level parsing of the Type 6 and Type 7 TLVs).

- TI E2E Forums for Chirp Tuning: e2e.ti.com (Search “IWR6843ISK GUI using people counting” to see TI engineer recommendations on profileCfg slopes for extended ranges).